# KRONES CONTRACTOR SAFETY MANUAL

TO: 		[INSERT CONTRACTOR]

DATE:		[INSERT DATE]

SUBJECT:	Contractors Safety Questionnaire and Safety Manual

At Krones we value the safety and health of our colleagues and suppliers. In looking for ways to improve our safety program, we have established a Contractor Safety Program. This document is part of the contractor qualification package for all companies providing services or working as contractors or subcontractors on our site located in Franklin, Wisconsin. The program intends to affirm your company’s commitment to comply with our safety requirements and applicable local, state, and federal laws and regulations.

All contactors are required to provide Krones with the following documentation:

- Current Certificate of Insurance with Krones as an Additional Certificate Holder

- Completed Contractor Questionnaire

- Signed Contractor Manual Acknowledgement Form

- Additional Information Upon Request

We would appreciate a timely response to ensure minimum interruptions to our business and to your project. Please return the Certificate of Insurance, completed questionnaire and signed acknowledgement form promptly to:

Krones, Inc.

Ms. Lara Riggins

9600 S. 58th Street

Franklin, WI  53132

Lara.Riggins@kronesusa.com

Thank-you for your help and support in making Krones a safer place to work. If there are any questions, please contact our Purchasing Department at (414) 409-4000.

Sincerely,

Krones, Inc.

James Criscimagna

Purchasing Manager

KRONES, INC.

CONTRACTOR QUESTIONNAIRE

| Company Name: |  |
| --- | --- |

| Address: |  |
| --- | --- |

| Name and Title of Safety Professional: |  |
| --- | --- |

| Telephone: |  | Email Address: |  |
| --- | --- | --- | --- |

| Total # of Full Time Employees: |  | Total # of Part-Time Employees: |  |
| --- | --- | --- | --- |

| SIC Code: |  | NAICS Code: |  |
| --- | --- | --- | --- |

OSHA 300 SAFETY INFORMATION

| Corporate Local Branch/Location | 2010 |  | 2011 |  | 2012 |
| --- | --- | --- | --- | --- | --- |

| A. Recordable Injury/Illness Cases (TCIR) (total of columns G through J on 300 log) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| B. Days Away Injury/Illness Cases (DAFWII) (total of column H on 300 log) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| C. Days Away, Restricted & Transfer Cases (DART) (total of columns H and I on 300 log) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| D. Number of Fatalities (total of column G on 300 log) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| E. Days Away from Work (total of column K on 300 log) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| F. Days of Job Transfer or Restrictions (total of column L on 300 log) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| G. Total Case Incident Rate (# of cases x 200,000 / total hours worked by all employees) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| H. DAFWII Rate (# of cases x 200,000 / total hours worked by all employees) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| I. DART Rate (# of cases x 200,000 / total hours worked by all employees) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

| J. Total Hours Worked by All Employees |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

Exempt from recordkeeping requirements.

EXPERIENCE MODIFICATION RATE (EMR)

List worker’s compensation Experience Modification Rate for the most recent 3 years and include documentation.

| Corporate Local Branch/Location | 2010 |  | 2011 |  | 2012 |
| --- | --- | --- | --- | --- | --- |

| Experienced Modification Rate (EMR) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |

*Attached a copy of the Certificate of Insurance with the organization named as an additional certificate holder.

REGULATORY CITATIONS

Has your company received any regulatory (OSHA, EPA, etc.) citations in the last 3 years?      Yes       No

*If yes, please explain.

SAFETY, HEALTH AND ENVIRONMENTAL PROGRAMS

Does your company have written safety, health and environmental program for the following:

| A. Management Commitment and Expectations | Yes No |
| --- | --- |

| B. Employee Participation | Yes No |
| --- | --- |

| C. Accountability and Responsibilities for All | Yes No |
| --- | --- |

| D. Weekly or Monthly Safety Meetings | Yes No |
| --- | --- |

| E. Hazard and Risk Recognition and Control | Yes No |
| --- | --- |

| F. Environmental Management System or ISO Certified* | Yes No |
| --- | --- |

*If ISO 14001-2008 certified, please attach a copy of certification.

SAFETY, HEALTH AND ENVIRONMENTAL WRITTEN PROCEDURES AND TRAINING

Does your company have written procedures and training for the following:

|  | Written Program |  | Training |  | Copy Provided |
| --- | --- | --- | --- | --- | --- |

| A. Safety Policy | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| B. Confined Space | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| C. Driving Safety | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| D. Electrical Safety | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| E. Lockout Tagout | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| F. Emergency Procedures | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| G. Environmental Policy | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| H. Fire Protection | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| I. First Aid | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| J. Hazard Communication | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| K. Housekeeping | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| L. Injury Reporting and Investigation | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| M. Job Safety Analysis | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| N. Material Safety Data Sheets | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| O. Personal Protective Equipment | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| P. Respiratory Protection | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| Q. RCRA Hazardous Waste | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| R. Spill Prevention and Control | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| S. Welding and Cutting | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| T. Work Hazard Recognition | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

| U. Others: | Yes No N/A |  | Yes No N/A |  |  |
| --- | --- | --- | --- | --- | --- |

*A copy of the program is only required when the work to be performed must follow specific program guidelines (i.e.

confined space entry, control of hazardous energy, hot work, etc.)

CERTIFCATION

I certify that the information provided on this form is complete and accurate. In addition, the organization will insure that all personnel sent to conduct work at Krones will be provided with and review the Contractor Safety Manual Rules and Guidelines.

| Name: |  | Title: |  |
| --- | --- | --- | --- |

| Signature: |  | Date: |  |
| --- | --- | --- | --- |

Internal Use Only Experience Mode Rate: . Year: . Acceptable Unacceptable Industry TRIR Rate: . Year: . Acceptable Unacceptable DART Rate: . Year: . Acceptable Unacceptable Additional Comments:

CONTRACTOR’S SAFETY MANUAL

RULES AND GUIDELINES

It is important that the Contractor Safety Program is made available to contractors as early as possible. The reasons include contractor awareness, agreement to comply, and to ensure any concerns can be addressed prior to execution of the work to be performed.

The Contractor’s Acknowledgement Form is part of the Krones’ Contractor Safety Program. If the nature of the project necessitates special safety and health requirements, these should be addressed separately.

It is mandatory that the contractor meets or exceeds the intent of the Contractor Safety Manual. This documentation does not release the contractor from their safety and health responsibilities or regulatory requirements. Non-compliance or violation with these rules could lead to the removal from site of the individual or contractor, as well as, termination of the contractor’s contract with Krones.

INTEGRATED MANAGEMENT SYSTEM POLICY

Quality

The prime target of our quality policy is to have entirely satisfied customers. To achieve this target, we strive to provide perfect-quality products and services, improve our processes, increase our productivity and thus ultimately lower our costs. A company’s employees and team can make or break the value and quality of a product or a service as each person is dependent on the knowledge, abilities and accuracy of the next. Thus, quality not only refers to the product quality, but more to the continual consideration of the customer’s requirements in all of the company sectors. Quality leads to satisfied customers. Satisfied customers lead to growth. Due to this high importance, we consider quality to be our main corporate target! At KRONES, quality has management backing!

Environmental Protection

KRONES is committed to its responsibility to protect the environment and keep it clean. This applies both to its own production and to the product KRONES offers its customers. With this in mind, the KRONES AG started up their enviro program. The entire product development is focused on ensuring that the customer saves energy and water and emits fewer harmful substances into the environment when using KRONES machines.

However, KRONES also aims to save on resources in its own factories and have as little impact on the environment as possible. This is achieved using new technological procedures and consistently implementing and observing the environmental regulations. To do so, the company sets itself environmental targets which allow it to pursue this subject in a structured and sustained manner.

Occupational Safety

The health and safety of its employees is the utmost priority of KRONES. Targeted programs contribute to the promotion of health. This is becoming increasingly important particularly in countries with an ageing population. In addition to this – also as part of the enviro program – KRONES AG is continually improving the safety of machines and lines and is designing more ergonomic work sequences. Uniform regulations regarding occupational safety, some of which have received independent certification in accordance with external standards, guarantee a high occupational safety level for our employees at our production sites. Employees from external companies are also intensively integrated in the existing training systems and are informed about KRONES occupational safety requirements. To allow us to evaluate the success of the measures, we integrated the employees from the external companies in our reports about occupational accidents at KRONES.

INTEGRATED MANAGEMENT SYSTEM POLICIES AND PROCEDURES

This is to advise you that the organization operates an Integrated Management System for quality, environmental, and health and safety for which specific policies and procedures have been established. As a result, depending on the work to be conducted on the behalf of the organization, additional training may be required.

SAFETY RULES AND REGULATIONS

It is the purpose of this manual to provide the Contractor with pertinent rules and regulations with their responsibilities while working in the facility or on our property. It shall be understood that all Contractors must comply with applicable Federal, State, and local laws and regulations in addition to these rules and regulations. Contractors are required to maintain their own safety and health programs, and at the same time comply with the intent of this Contractor Safety Manual rules and regulations contained within this document.

GENERAL SAFETY

Conduct

Contractors are required to conduct themselves in a proper manner at all times while at the organization or on its premises. Fighting, gambling, horseplay, stealing, sleeping, destruction of property, abusive language, disrespectful behavior, and other activities, which may lead to unsafe conditions, are prohibited. Violations of this requirement will result in the immediate removal of the violating individual from the premise and may result in the termination of the contractor’s contract.

Eating and Drinking

Eating and drinking is only permitted in designated areas on the property. Cafeteria and break rooms are available for use by contracted personnel. Drinking water is available by water fountains (i.e. bubblers) located throughout the site.

Firearms and Weapons

The possession of firearms or other weapons are expressly prohibited on company property. Violation of this requirement will result in the immediate removal of the violating individual from the premises.

Smoking

Smoking is not permitted within the workplace and is only allowed in designated outside areas. Smoking is limited to break times and lunch hours only.

Substance Abuse

The possession or drinking of alcoholic beverages and the possession or use of illegal drugs is expressly prohibited while on the property. Violation of this requirement will result in the immediate removal of the violating individual from the premises.

Toilets

The toilets and wash facilities on the property are available and may be used by contractor personnel.

Audio Equipment

The uses of audio equipment, including radio headsets, are not permitted for use on the property unless authorized by the organization.

Driving and Parking of Vehicles

Contractors shall park in designated parking areas and must meet state laws and have a valid license plate. Parking outside of designated parking areas must be authorized by the organization. Accidents involving vehicles shall be reported to Security.

Solicitation and Distribution

Solicitation and distribution is not allowed at the organization.

Confidentiality

Photos or videotaping is strictly prohibited at the organization. Any exemption to this rule needs to be authorized in writing by the organization.

Visitor Passes

Contractors are required to register with the Receptionist/Security on a daily basis. Each contracted person will be issued a visitor badge for the day. The visitor badge must be worn and the contracted person must sign-out and return the badge at the end of each work shift.

Security

Security personnel stationed on the premises are authorized to inspect clothing, vehicles, briefcases, packages, tools, equipment, or any other container on the property at anytime, upon entering or leaving the property.

EMERGENCIES

Planning

Prior to the start of work, the Contractor will provide the organization with an emergency plan which shall include emergency procedures. The contractor is responsible for ensuring their personnel are aware of the procedures and information pertaining to emergency telephone numbers, first aid personnel, local hospitals and ambulance service.

Notification

In the event of an emergency that involves personnel, equipment, or facilities, the Contractor should immediately contact the Receptionist/Security to advise the organization of the event and include the contractor’s name, type of emergency, location of the emergency and any persons injured. The organization will contact and arrange for appropriate assistant as warranted.

Evacuation

In the event the emergency alarm is initiated, the Contractor is required to stop work and to leave the area in an orderly fashion. The Contractor should proceed to the predetermined location outside of the building and remain there until the return to work is authorized by the organization.

SAFETY PRACTICES

Housekeeping

The work site must be kept clean and orderly at all times. When applicable, small tools and equipment must be stored properly at the end of each work shift. At the end of each shift, scrap materials should be removed and properly disposed in the designated receptacle container. When construction work is being performed, the area will have a physical barrier from other plant areas. The contractor is responsible for the security of his tools, equipment, and materials.

Unobstructed Areas

Aisles and stairwells must be kept free from obstruction. Items should not be stored in front of doors, exits, or other means of egress. Fire extinguishers should not be blocked and electrical panels must maintain a 36 inches clearance.

Work on Fire Alarm or Protection Systems

If a contractor’s work requires that the fire alarm or sprinkler system be put out of service, the contractor is required to submit a written request to the organization no less than 24 hours prior to the time the system is scheduled to be interrupted. The interruption must be in accordance with company procedures and the work must be performed to ensure that the fire alarm and sprinkler systems are returned to service by the end of the work shift.

Vapor, Fume and Dust Operations

Contractors will coordinate with the organization to plan for operations producing fumes, vapors, or dust (such as painting, coating, etc.) with a minimum of disruption to organization’s facilities, employees, and operations. Under no circumstances will such operations be conducted without the prior approval of the organization. Contractors will provide proper ventilation in enclosed spaces during the application of these materials and will provide their personnel with respirators, as required, for the safe application of the material.

ACCIDENTS AND INJURIES

In the event that accident occurs that requires first aid, the Contractor should immediately contact the Receptionist/Security to advise the organization of the nature, location, and severity of the incident. The organization will summons the on-site First Aid Team to respond. If a minor injury, the First Aid Team will aid the injured personnel. If the injured personnel requires treatment beyond first aid, the organization shall arrange for additional medical assistance.

Bloodborne Pathogens

Blood and other bodily fluids that become present as the result of an accident or injury will be properly cleaned-up by the First Aid Team in accordance with the organization’s Bloodborne Pathogen Program.

Incident Investigation

The Contractor is required to perform an incident investigation and take appropriate corrective measures to prevent a reoccurrence of the accident. The Contractors will follow their own internal investigation procedures. A copy of the incident report, along with the corrective action taken, is required to be provided to organization upon request.

OSHA Recordkeeping

The Contractor is responsible for tracking the total hours worked, accident occurrence, frequency, severity, restricted work days, days away from work, and maintain the OSHA 300 Log. Records may be requested.

PERSONAL PROTECTIVE EQUIPMENT

The Contractor will use the personal protective equipment needed for the job per federal regulations and company requirements. The contractor is responsible for providing their own personal protective equipment. Any exemption to this rule needs to be authorized in writing by the organization.

Attire

Contractors are required to wear shirts and long pants when performing work on the premises. In designated areas, contractors may be required to wear additional protective garments (e.g. apron, hair covering, etc.).

Eye and Face Protection

Safety glasses with permanent side-shields, which meet the ANSI Z87.1 specifications, must be worn by the Contractor while performing work on the premises. Additional eye and face protection will be required to be worn when exposed to flying particles, molten metal, liquid chemicals, light radiation or other hazardous conditions were additional protection is required.

Foot Protection

Steel-toe safety footwear is required to be worn by the Contractor when performing work on the premises. Contractors performing electrical work are required to wear electrical hazard protective footwear.

Hand Protection

Contractors are required to ensure their personnel are provided with proper gloves whenever hands are exposed to chemicals or other hazardous conditions. Contractors may use general work gloves, leather gloves, chemical-resistant gloves, or other appropriate hand protection.

Head Protection

Approved safety hard hats meeting ANSI Z89-1986 standards are required to be worn on projects involving multi-layer or overhead work. Metal hats are prohibited.

Hearing Protection

Contractors are required to ensure their personnel are provided with proper hearing protection devices whenever noise levels exceed established threshold set forth by federal standards. Contractors may use either disposable earplugs or ear muffs with suitable sound attenuation rating for the work to be performed. Hearing protection must be provided at all times for personnel exposed to sound pressure levels exceeding 85 dBA. If reusable equipment is provided, appropriate storage and cleaning facilities must be provided.

Respiratory Protection

Contractors will ensure that their personnel are provided with NIOSH approved respirators when performing any dust or fume producing operations or whenever the individual comes into contact with material whose manufacturer recommends the use of respirator during application of the product. Respirators shall be inspected on a daily basis to ensure proper maintenance and operation. Contractors may be required to supply documentation verifying that each person wearing a half-mask air-purifying or air-supplied respirators has had medical clearance and was properly fit tested.

# Body Protection

Contractors performing electrical work are required to wear the appropriate level of flame retardant clothing required for the work to be performed in compliance with NFPA 70E standard.

CONFINED SPACE ENTRY

## Contractors conducting confined space entry are required to comply with the requirements set fourth under 29 CFR 1910.146 governing confined space entry. The contractors are responsible for supplying their own monitoring and retrieval equipment. The confined space entry permit must be complete and visibly posted in the work area. The contractor is also responsible for providing the organization with a copy of their Confined Space Program and documentation verifying that each person conducting confined space entry has been properly trained.

HOT WORK

## Contractors conducting hot work, such as cutting, welding, brazing, etc, are required to comply with the requirements set fourth under 29 CFR Subpart Q governing hot work. A hot work permit is required to be issued by the organization for the specific activity and for a time period of not more than one work shift. Permits are required to be prominently displayed in the location for which the work is being conducted. The organization is responsible for canceling the hot work permit once the work is completed.

LOCKOUT TAGOUT

## When processes and plant equipment must be made inoperable by the Contractor prior to performing work, the Contractor must receive approval by the organization before conducting lockout/tagout procedures. The lockout and tagging of the equipment must be performed jointly by the contractor and the organization and the locks should readily identify the individual(s) locking out the equipment. The organization’s removal procedure should be strictly adhered to when removing a lock by someone other than the individual assigned to the lock. Contractors may be required to provide a copy of the contractor’s Lockout Tagout Program and documentation verifying that each person conducting lockout and tagout operations has been properly trained.

POWERED EQUIPIMENT

Contractors are required to furnish their own equipment. The restriction includes lifts, powered industrial trucks, hoists, and cranes. Any exemption to this rule needs to be authorized in writing by the organization.

Aerial and Scissor Lifts

Contractors that utilize our high lift equipment or bring their own high lift equipment on-site to be used must be trained and certified in accordance with OSHA and ANSI standards. Contractors that can not demonstrate compliance with the training requirements will not be allowed to operate our equipment. In addition, personnel on-site will be required to demonstrate compliance with the training requirements by presenting their Certificate of Completion Card or other documentation of training prior to operation.

Cranes and Hoists

Contractors are prohibited from operating cranes and hoists present on the premises. Exemption from this rules includes a Contractor that has been contracted to provide inspections or training. Any other exemption to this rule needs to be authorized in writing by the organization. Contractors may be required to supply documentation verifying that each person operation such equipment has been properly trained.

Powered Industrial Trucks

Contractors that utilize our powered industrial trucks or bring their own powered industrial trucks on-site to be used must be trained and certified in accordance with OSHA standards. Contractors that can not demonstrate compliance with the training requirements will not be allowed to operate our equipment. In addition, personnel on-site will be required to demonstrate compliance with the training requirements by presenting their Certificate of Completion Card or other documentation of training prior to operation.

HAZARD COMMUNICATION

## Contractors are required to provide the organizations with Material Safety Data Sheets for all chemical brought on the premises prior to performing work upon request. The chemical containers must be properly labeled to its contents and properly stored when not in use. Contractor personnel are required to wear the appropriate personal protective equipment when handling chemicals. Spill of chemical should be immediately reported to the organization for proper clean-up and disposal.

In addition, Contractors may be exposed to chemicals as the result of performing work on our premises. Copies of the Material Safety Data Sheet are available upon written request by the contractor.

EQUIPMENT

Compressed Air

Compressed air may be used for cleaning or servicing of equipment, where appropriate. When using compressed air, the Contractor must ensure the use of a pressure reducing nozzle or reduce the pressure to 30 psi or lower.

Electrical Equipment

It is the contractor’s responsibility to ensure all electrical work performed complies with the applicable National Electrical Code and NFPA 70E requirements. All electrical equipment must be in good condition and properly grounded. In the event that electrical equipment is found defective, it must be taken out of service and repaired prior to use.

Extension Cords

Temporary electrical cords must be covered or elevated in locations where they are subject to vehicle damage, submersed in water, or in areas of interference with people or construction. Energized wiring in outlets, junction boxes, circuit breaker panels, and other devices must be covered at all times to protect against accidental contact.

Fall Protection

Contractors working from unguarded heights, on steep slopes, mechanical lift equipment, or otherwise subjected to falls above 6 feet shall be secured by safety harnesses and lifelines. Safety harness and lifelines shall be inspected daily and repaired if found defective, prior to work continuing. Guardrails may be considered satisfactory protection as long as they completely remove the danger of falling under normal situations and are installed according to code.

Gas Cylinders

A suitable gas cylinder truck, chain, or other secure fastening method shall be used to keep cylinders from falling or being knocked over while in use. Oxygen and acetylene cylinders must not be stored within 20 feet or one another unless they are separated by a 30-minute non-combustible barrier. When cylinders are being transported, they must be held securely and valve protection caps must be in place. During cutting or burning operation, a fire extinguisher must be stationed at each gas cylinder cart.

Heavy Equipment

Heavy equipment must be in a safety mechanical working condition. The Contractor is responsible for inspection the equipment to ensure its safe operation condition. Defective equipment must be removed from service immediately. Operators must be trained and qualified to operate the type of equipment for which they are assigned. Documentation of the operator’s training may be required to be provided to the organization upon request.

Ladders

Ladders are required to be inspected prior to each use. Ladders found to be defected shall be not be used and removed from premises. Ladders shall be used in accordance with the manufacturer’s requirements and only for the purpose of which they are intended.

Scaffolding

Scaffolds shall be constructed of sound material and properly supported on a firm footing (not concrete blocks or loose material). All scaffolding must meet code requirements. Scaffolding must be equipped with standard handrail, mid-rail, and toe-board, and shall be tied to an adjacent wall, both vertically and horizontally. Fall protection will be required when the use of handrails is not possible. Under no circumstances are contractor’s personnel allowed to ride on mobile scaffolds.

Signs and Barriers

Contractors shall use signs and barriers to isolate hazardous areas. When used outside the building, the appropriate warning lighting shall be used.

TOOLS

Contractor owned tools may be subject to inspection by the organization. Defective tools must be immediately removed from service. Tools should be locked up after each work shift. Lost or stolen tools should be reported to the organization.

Actuated Powered Tools

The use of actuated power tools is prohibited unless its use is authorized in writing by the organization.

Electrical Tools

The metal non-current carrying parts of all electrical tools must be grounded. Contractors shall use ground fault interrupt circuits or the assured equipment grounding conductor program. Under no circumstances will electrical powered tools be hoisted or lowered by their electrical cords.

Pneumatic Powered Tools

Pneumatic powered tools shall be inspected prior to each use. Hose lines shall be placed to eliminate a tripping hazard and to minimize traffic over the lines. Prior to disconnecting the tool from the airlines, pressure must be shut-off and exhausted from the line. Safety clips, chains, wires, or other retainers shall be used to secure the hose together and to secure the hose to the power source, and the tool to the hose.

Abrasive Grinding Tools

Abrasive grinding tools shall be inspected prior to each use. Abrasive wheels must be kept dressed. Excessively worn or damaged wheels shall be replaced prior to use. Standard guarding will be installed and used on grinding wheels in accordance with manufacturer’s specifications. Work rests shall be adjusted close to the wheel with a maximum opening of 1/8”. The tongue shall be adjusted close to the wheel with a maximum opening of ¼”.

ENVIRONMENTAL

Regulated Waste

Contractors are responsibility to notify the organization of any waste produced or collected, which may be characterized as a regulated waste. If there are questions regarding the characteristics or waste disposal methods required, the organization should be contacted for assistance. Waste sampling and disposal costs are the responsibility of the contractor unless a specific agreement is made between the Contractor and organization prior to the waste being produced.

Recyclable Materials

The organization recycles cardboard, metal, aluminum, glass, and various other materials. Contractors are required to recycle these material, as well, and is prohibited from placing these material in the solid waste dumpster for general disposal. The use of the organization’s recyclable containers is not allowed unless a specific agreement is made between the Contractor and organization prior to the material being produced.

Solid Waste

Contractors are responsible for the clean-up and proper disposal of all general refuse debris prior to the end of each work shift. The use of the organization’s general refuse container is not allowed unless a specific agreement is made between the Contractor and Krones prior to the general refuse debris being produced.

Spill Clean-Up

In the event that a chemical spill should occur that requires clean-up response, the contactor personnel should immediately notify the organization and advise of the nature, location, and severity of the spill. The organization will notify the Spill Response Team. If a minor spill, the Spill Response Team will clean-up the material. If the spill requires additional assistance, the organization will make arrangements for additional outside emergency response assistance. If additional spill response assistance is necessary, it shall be at the Contractor’s expense.

SUBCONTRACTORS

The contractor is required to notify the organization of any subcontractors that will be on-site on their behalf prior to work being performed. The contractor is responsible for the sub-contractor while at the organization. This includes ensuring that the subcontractor complies with all federal, state, and local regulations and the organization’s Contractor Safety Manual.

CONTRACTOR ACKNOWLEDGEMENT FORM

We hereby are committed to have our company comply with Krones’ Contractor Safety Manual requirements and with all local, state, and federal laws and regulations, as applicable. I acknowledge that the below reference company has received a copy of the Krones Contractor Safety Manual and that we are responsible to read and abide by the safety requirements set fourth within this agreement.

Variances from the rules contained within this document, which requires written approval by Krones, should be attached as an addendum to this document.

COMPANY:

| Company Name: |  |
| --- | --- |

| Company Address: |  |
| --- | --- |

| City, State, Zip Code: |  |
| --- | --- |

| Contact: |  |
| --- | --- |

| Telephone Number: |  |
| --- | --- |

| Contractor Representative Officer of Designee(1) (Name and Signature) |  | Date |
| --- | --- | --- |